# Individual Project Report: IoT Rain Detector

Student Name: Ahmad Muhay Ud Din

Student ID: S00287223@atu.ie

Course: Iot Project 1

Team Members: Ahmad Din, Arif Taha, Joshua Jameson, Paschal Okolo

### Introduction:

### The Rain Detector project is a collaborative effort from my team and by me aimed at building an integrated IoT system to monitor rainfall and ambient conditions. Unlike a traditional delegated structure, our team operated on a Total collaboration model, where every member is involved in the hardware assembly, coding, testing the software and hardware and documentation phases. This approach ensured that each of us has a comprehensive understanding of the entire system of our project.

### My Contributions in the Collaborative Process

Because we worked as a unit, my contributions spanned the entire development pipeline:

- Hardware Integration & Prototyping: I participated in the physical session where we connect rain sensor, temperature sensor, Light sensor, led, OLED display to the Arduino. Working together allow us to double check wiring the Arduino Kit to the sensors and it results in the reducing the risk of hardware failure.

  - Collaborative Coding & Debugging: During the programming phase of Arduino for sensors, I worked alongside my teammates to calibrate the sensor thresholds. We collectively decided to use analog reading (0 – 1023) to create granular detection system rather than a simple digital High/Low.

- Problem Solving (The Temperature Issue): When we encountered the incorrect temperature readings, I worked with the group to cross-reference the sensor version (v1.3) with the code constants. We collectively identified that changing the 'B' value to 3950 solved the discrepancy.

- ​Sensor Version Mismatch: The temperature sensor initially failed because the provided sample code was for version 1.2, while the hardware was version 1.3. The team identified the issue and updated the constant value B from 4275000 to 3950 to achieve accurate readings.

- ​Display Dependency: It was discovered that the project loop slowed significantly (approximately 5 seconds per update) if the OLED display was disconnected. This was attributed to the library timing out while waiting for a response from the missing hardware.

- ​Threshold Consistency: Testing revealed fluctuating dry-state readings across different days, necessitating ongoing investigation to fine-tune threshold values for real-world rain detection.

- Documentation & Version Control: I contributed to our shared GitHub repository and Trello board, ensuring that our progress was synchronized and that our "Team Report" accurately reflected our collective findings.

### Key Technical Challenges Overcome

Working closely with the team allowed us to solve complex issues faster:

- I2C Latency: We discovered together that the code slowed down significantly if the OLED was unplugged. This collective observation led us to research how the I2C library handles timeouts.

- Calibration: We spent time as a group testing the rain sensor with actual water drops to find the "sweet spot" for the buzzer and LED alerts, ensuring the system wasn't too sensitive to humidity but responsive to rain.

### Personal Reflection on Teamwork:

I gained experience in areas I might have otherwise avoided. I improved my understanding of:

- Hardware-Software Interfacing: How physical moisture is converted into a data point.

- Agile Management: Using Trello as a group to move tasks from "To-Do" to "Done."

- Security Literacy: Discussing as a group how we could encrypt our sensor data (AES) and move to HTTPS for cloud logging.

### Conclusion:

The "Rain Detector" project was a success not just in its technical execution, but in its collaborative delivery. By contributing to every stage—from the first wire connection to the demo day GitHub commit—I have developed a view of IoT development that my team and I will carry into future modules.